Duel Tanks

Sathvik Yechuri

Duel Tanks is a real-time two-player tank battle game played across two separate MSPM0G3507 LaunchPads. Each board will be connected to its own ST7735 LCD display and linked via UART. Each player controls a tank on their own screen using a slide potentiometer to rotate the tank's turret and two external buttons to fire projectiles and even enable barrier walls as defense. The goal is to destroy the opponent's tank by landing projectile hits before they land three hits on you. The players start with three lives shown on screen, and when a shell reaches the edge, its data is sent via UART and appears on the opponent’s screen. Each hit triggers an explosion, sound, and decrements lives via interrupts. When a player reaches zero lives, both screens display the winner! The game supports English and Spanish, and I’m planning to use a custom PCB that integrates the LaunchPad, inputs, LCD, UART, and DAC speaker.

Features:

- Slide potentiometer on each PCB rotates tank turret

- Button 1: fire a projectile shell at current turret angle

- Button 2: deploy a temporary barrier wall

- Cross-screen projectiles sync: shells are transmitted over UART to appear on opponent's display

- Sounds: cannon fire, shield, explosion on hit, victory fanfare

- Lives displayed as heart icons on each screen; synchronized between boards via UART

- Score (kill count) displayed in corner of each screen throughout the game

- Winner and loser screens displayed on both boards simultaneously at game end

- Language selection screen on startup: English and Spanish

Requirements met:

- Two buttons: fire projectile and deploy barrier wall, both with hardware debouncing

- Slide pot: rotate turret angle

- 3+ sprites: Tank body, turret, bullets, explosion, and barrier wall sprites

- Four sounds with 12-bit DAC: cannon fire, barrier deploy, explosion, victory fanfare

- Score and lives always displayed on screen during gameplay

- Three interrupt ISRs: Timer A (game loop + ADC sample), Timer B (DAC audio), GPIO ISR (button press events)

- UART communication between two MSPM0

- Two languages: English and Spanish